熱傳學期末考(103/01/10)

- The vertical rear window of an automobile, as shown in Fig. 1, is of thickness L= 8 mm and height H=0.5 m and contains fine-meshed heating wires that can induce nearly uniform volumetric heating,  (W/m3). Consider steady-state conditions for which the interior surface of the window is exposed to quiescent air at 10 oC, while the exterior surface is exposed to air at –10 oC moving in parallel flow over the surface with a velocity of 20 m/s. Determine the volumetric heating rate needed to maintain the interior window surface at Ts,i= 15 oC.

Note: Glass (300 K): k = 1.4 W/m-K; Air (12.5°C, 1 atm):  ν =14.6 × 10-6 m2/s, k = 0.0251 W/mK, α = 20.59 × 10-6 m2/s, β = 3.503 × 10-3 K-1,

Pr = 0.711; Air (0°C): ν = 13.49 × 10-6 m2/s, k = 0.0241 W/mK,

Pr = 0.714.

Fig. 1

Correlations of average Nusselt number:

- Forced convection over a flat plate:

(i) Laminar flow:

(ii) Turbulent flow:

(iii) For the combined laminar and turbulent conditions:

Rex,c=5x105

(2) Natural convection along a vertical plate over the entire range of RaL:

- As shown in Fig. 2, water flows through a thick-walled tube with an inner diameter of 12 mm and a length of 8 m. The tube is immersed in a well-stirred, hot reaction tank maintained at 85oC, and the conduction resistance of the tube wall (based on the inner surface area) is =0.002 m2K/W. The inlet temperature of the process fluid is Tm,i= 20oC, and the flow rate is 33kg/h. Assume fully developed flow and thermal conditions within the tube. You may neglect the convective thermal resistance of the well-stirred, hot reaction fluid. Please calculate the outlet temperature of the process fluid, Tm,o.

Note: Properties of water: k= 0.634 W/mK, Cp= 4179 J/kgK,

=6.31x10-4 Ns/m2, Pr=4.16.

Fig. 2

Correlations of average Nusselt number in a circular tube: (constant surface temperature)

- Laminar flow: = 3.66

- Turbulent flow:

- A 6-m-long section of an 8-cm-diameter horizontal hot-water pipe shown in Fig. 3 passes through a large room whose temperature is 20oC. If the outer surface temperature of the pipe is 70oC, determine the rate of heat loss from the pipe by natural convection.

Note: the properties of the air at 45oC and 1 atm are k= 0.02699 W/mK,

= 1.750x10-5 m2/s, Pr=0.7241.

The empirical correlation for the average Nusselt number for natural convection over a horizontal cylinder:

1012

Fig. 3